Table of Contents

Creating Register Detail Files for EST’s  SIM Products (PC and UNIX)

For Internal Use Only

Rev. 3.0

Introduction

Version Document

## Introduction

The powerful SIM internal register configuration utility is available on both the PC (visionSIM) and UNIX (visionXP / visionXD SIM) platforms.

This document describes how EST SIM Register Detail Files (.DTL) and associated help files (.HLP or .help) are created, modified, and tested.

The purpose of these SIM related files is to provide the setup, configuration, and information required by the SIM products to properly display SIM register groups, bit-level details, and context sensitive help.

The various files utilized by SIM include:

- A Register Detail File (.DTL)

- A Register File (.REG)

- A Symbol File (.SYM – UNIX only)

- A Help File (.HLP for PC, .help for UNIX)

## File Naming Conventions

### Detail Files

For register detail files, 68K and ColdFire microprocessor family files are named by processor, as follows:

PC:  68332.DTL, MCF5206.DTL
UNIX:  reg360.dtl, reg5307.dtl

The UNIX version of these files are named with “REG” plus the three or four-digit processor number.

For the PowerPC family, the naming convention is slightly different for the PC only  When the detail files were initially created, it was determined that the various processors shared many of the same register groups.  In an effort to streamline the detail file creation process, we created .DTL files based on register group names, rather than processor names, as follows:

PC:  MPC8COMM.DTL, MPC8IOPO.DTL

On UNIX, PowerPC family detail files are named similarly to the 68K and ColdFire families, as follows, again beginning with “REG”:

UNIX:  reg850.dtl, reg860.dtl

Note:    UNIX is case sensitive, so it is important that you name your files as indicated above, with no capitalization.

### Register Files

The Register File (.REG) is a simple text file containing a list of register files, organized by group; register group names; and register address information.  This same listing displays when you type an SC (System Configuration) emulator command at a >BKM> prompt.

Files are named defprocessorname.reg, where processorname is the three or four digit processor number, for both the PC and UNIX platforms.  For example:

PC:  DEF340.REG, DEF821.REG, DEF5204.REG
	UNIX:  def340.reg, def821.reg, def5204.reg

Note:    UNIX is case sensitive, so it is important that you name your files as indicated above, with no capitalization.

### Symbol Files

For the UNIX platform only, a symbol file (.SYM) is also required for the proper display of register information in the SIM utility.  The symbol file is a simple text file that contains a list of pin symbols for the associated processor.

Files are named in the same convention as the register files, except that they have an .SYM extension, as follows:

PC:  DEF332.SYM, DEF850.SYM, DEF5307.SYM
	UNIX:  def332.sym, def850.sym, def5307.sym

Note:    UNIX is case sensitive, so it is important that you name your files as indicated above, with no capitalization.

### Help Files

Both the PC and UNIX versions of the SIM product have associated help files that provide bit-level detail for every register.

On the PC, the help file is created using RoboHelp or another WinHelp GUI compiler.  Files are named processorname.HLP, where processorname refers to the name of the processor.  For example:

PC:  68332.HLP, MPC821.HLP, MCF5204.HLP

On UNIX, the help file is created as a simple text file using a text editor.  Files are named using the following convention:  Cpu.processorname.help, where processorname refers to the name of the processor.  For example:

UNIX:  Cpu.68332.help, Cpu.mpc860.help, Cpu.mcf5206.help

Note:    UNIX is case sensitive, so it is important that you name your files as indicated above, with proper capitalization where necessary.

## File Examples

The next few pages provide some examples of the content of the various SIM-related files.  You can also use a text editor to open any .DTL, .REG, .SYM, or .help file on your hard drive.  PC help files must be viewed using the WinHelp interface.

### Detail File Example

#### PC Detail File

The following is an excerpt from a PC .DTL file named MPCIOPO.DTL (remember, PowerPC detail files are grouped and named based on register group name).  Note that any line beginning with a pound sign (#) is a comment and is not processed by SIM.

# EMBEDDED SUPPORT TOOLS CORPORATION

# COPYRIGHT 1996-1998.  ALL RIGHTS RESERVED.

#

# DTL GROUP DESCRIPTION: IOPORTS GROUP

#

# PROCESSORS:   801 POWERPC

#		          821 POWERPC

#                             823 POWERPC

#                             860 POWERPC, 860T, 860SAR

#

# MOTOROLA MANUAL:  MPC860 User's Manual and MPC801 User's Manual

#

# REVISION	DATE		DESCRIPTION

# --------	----		-----------

# 1.0	10-6-96  	               Original

# 1.1	10-28-96	               1. Replaced PAODR with PXODR.

#			      	2. Added EDMX bit-level help for PCINT register.

# 1.2              11-8-96                 1. Added ODx bits to PXODR and added context numbers.

# 1.3              11-11-96               1. Re-ordered bits of PCSO register so they appear in order in grid.

#                                                 2. Re-named EXTx bits to DREQx for the PCSO register.

# 1.4              12-03-96               1. Fixed syntax errors for Port X registers.

# 1.5              05-23-97               1. Added Port B registers for MPC801

# 1.6	 01-21-98	1. Corrected options for PBODR register.

# 1.7              03-04-98               1. Added PDPAR shadow register for 860 SAR

# 1.8	 03-13-98	1. Modified context ID nos./added comment help indices for SAR

# ---------------------------------

REVISION,1.8,IOPORTS Group

HELPINDEX,IOPORTS_PXODR_HELP,100

HELPINDEX,IOPORTS_PXODR_OD9_HELP,102

HELPINDEX,IOPORTS_PXODR_OD10_HELP,104

HELPINDEX,IOPORTS_PXODR_OD11_HELP,106

HELPINDEX,IOPORTS_PXODR_OD12_HELP,108

HELPINDEX,IOPORTS_PXODR_OD14_HELP,110

HELPINDEX,IOPORTS_PXDAT_HELP,200

HELPINDEX,IOPORTS_PXDIR_HELP,300

HELPINDEX,IOPORTS_PXPAR_HELP,400

# HELPINDEX,IOPORTS_PXPAR_ATM_HELP,402

# HELPINDEX,IOPORTS_PXPAR_UT_HELP,404

# HELPINDEX,IOPORTS_PXPAR_HELP,406

HELPINDEX,IOPORTS_PCSO_HELP,500

HELPINDEX,IOPORTS_PCSO_CDX_HELP,502

HELPINDEX,IOPORTS_PCSO_CTSX_HELP,504

HELPINDEX,IOPORTS_PCSO_DREQX_HELP,506

HELPINDEX,IOPORTS_PCINT_HELP,600

HELPINDEX,IOPORTS_PCINT_EDMX_HELP,602

HELPINDEX,IOPORTS_PBODR_HELP,700

HELPINDEX,IOPORTS_PBODR_ODX,702

HELPINDEX,IOPORTS_PBDAT_HELP,800

HELPINDEX,IOPORTS_PBDAT_DX_HELP,802

HELPINDEX,IOPORTS_PBDIR_HELP,900

HELPINDEX,IOPORTS_PBDIR_DRX_HELP,902

HELPINDEX,IOPORTS_PBPAR_HELP,1000

HELPINDEX,IOPORTS_PBPAR_DDX_HELP,1002

# described in section 16.19.4.3 page 16-457 of the MPC860 UM

(PADIR),1,Port A Data Direction Register(0-in/1-out),IOPORTS,IOPORTS_PXDIR_HELP

T,Data Direction,1,16,Binary,IOPORTS_PXDIR_HELP,

# described in section 16.19.4.4 page 16-457 of the MPC860 UM

(PAPAR),1,Port A Assignment Register,IOPORTS,IOPORTS_PXPAR_HELP

T,Pin Assignment,1,16,Binary,IOPORTS_PXPAR_HELP,

# described in section 16.19.4.1 page 16-456  of the MPC860 UM

(PAODR),5,Port A Open-Drain Register,IOPORTS,IOPORTS_PXODR_HELP

O,OD9 Bit,10,1,I/O pin is open-drain driver,I/O pin is actively driven output,IOPORTS_PXODR_OD9_HELP,

O,OD10 Bit,11,1,I/O pin is open-drain driver,I/O pin is actively driven output,IOPORTS_PXODR_OD10_HELP,

O,OD11 Bit,12,1,I/O pin is open-drain driver,I/O pin is actively driven output,IOPORTS_PXODR_OD11_HELP,

O,OD12 Bit,13,1,I/O pin is open-drain driver,I/O pin is actively driven output,IOPORTS_PXODR_OD12_HELP,

O,OD14 Bit,15,1,I/O pin is open-drain driver,I/O pin is actively driven output,IOPORTS_PXODR_OD14_HELP,

# described in section 16.19.4.2 page 16-456 of the MPC860 UM

(PADAT),1,Port A Data Register,IOPORTS,IOPORTS_PXDAT_HELP

T,Port A Data,1,16,Binary,IOPORTS,IOPORTS_PXDAT_HELP,

# described in section 16.19.10.2 page 16-468 of the MPC860 UM

(PCDIR),1,Port C Data Direction Register(0-in/1-out),IOPORTS,IOPORTS_PXDIR_HELP

T,Data Direction,1,16,Binary,IOPORTS,IOPORTS_PXDIR_HELP,

# described in section 16.19.10.3 page 16-468 of the MPC860 UM

(PCPAR),1,Port C Assignment Register,IOPORTS,IOPORTS_PXPAR_HELP

T,Pin Assignment,1,16,Binary,IOPORTS_PXPAR_HELP,

# described in section 16.19.10.1 page 16-468 of the MPC860 UM

(PCDAT),1,Port C Data Register,IOPORTS,IOPORTS_PXDAT_HELP

T,Port C Data Register,5,12,Binary,IOPORTS_PXDAT_HELP,

#### UNIX Detail File

The following is an excerpt from a UNIX .DTL file named reg860.dtl (remember, UNIX is case sensitive).  UNIX detail files do not currently allow comments.

ESTDTAIL

CSVIEW,14

600,8,CS0,CS1,CS2,CS3,CS4,CS5,CS6,CS7,

850,BaseAddr,0,1,17,L,8,

850,AddrMsk.,1,1,16,L,8,

300,AT,0,18,1,B,

450,ATM,1,18,2,B,

600,Size,0,21,2,C,4,32-Bit,8-Bit,16-Bit,Res,

400,Prty,0,23,1,O,off,on,

400,WP,0,24,1,O,off,on,

500,Valid,0,32,1,O,no,yes,

400,Cyc,1,25,4,C,16,0,1,2,3,4,5,6,7,8,9,10,11,12,13,14,15,

500,Burst,1,24,1,O,off,on,

600,MS,0,25,2,C,4,GPCM,UPMA,UPMB,Res,

450,Ack,1,29,1,O,int,ext,

BR0,OR0,BR1,OR1,BR2,OR2,BR3,OR3,BR4,OR4,BR5,OR5,BR6,OR6,BR7,OR7,

CSDTL,15

T,0,Base Addr.,1,16,LeftHex,8,

T,0,AT,17,2,Binary,

C,0,Size,21,2,4,32-Bit,8-Bit,16-Bit,Res,

O,0,Parity,23,1,Enabled,Disabled,

O,0,Write Protect,24,1,Enabled,Disabled,

C,0,MS,25,2,4,GPCM,UPMA,UPMB,Res,

O,0,Valid Bit,32,1,Valid,Invalid,

T,1,Msk. Addr.,1,16,LeftHex,8,

T,1,AT,17,2,Binary,

O,1,CSNT/SAM,21,1,Special,Normal,

C,1,ACS,22,2,4,CS half,Res,CS quarter,CS normal,

O,1,Burst,24,1,Enabled,Disabled,

C,1,Cyc.,25,4,16,0 wait,1 waits,2 waits,3 waits,4 waits,5 waits,6 waits,7 waits,8 waits,9 waits,10 waits,11 waits,12 waits,13 waits,14 waits,15 waits,

O,1,SETA,29,1,Enabled,Disabled,

O,1,TRLX,30,1,Enabled,Disabled,

(IMMR),3,Internal Memory Map Register

T,ISB-Internal Space Base,1,16,Binary,

T,PARTNUM-Part Number,17,8,Binary,

T,MASKNUM-Msk. Number,25,8,Binary,

### Register File Example

#### PC Register File

The following is an excerpt from a PC .REG file named DEF5206.REG.  Note that any line beginning with a pound sign (#) is a comment and is not processed by SIM.

SC   MBAR        00000C0F       10000001 SIMR

SC   RAMBAR   00000C04       00000000 SIMR

SC   SIMR          10000003       C0            SIMR

SC   ICR1          10000014         04            SIMR

SC   ICR2          10000015         08            SIMR

SC   ICR3          10000016         0C            SIMR

SC   ICR4          10000017  10       SIMR

SC   ICR5          10000018  14       SIMR

SC   ICR6          10000019  18       SIMR

SC   ICR7          1000001A  1C       SIMR

SC   ICR8          1000001B  1C       SIMR

SC   ICR9          1000001C  80       SIMR

SC   ICR10         1000001D  80       SIMR

SC   ICR11         1000001E  80       SIMR

SC   ICR12         1000001F  00       SIMR

SC   ICR13         10000020  00       SIMR

SC   IMR           10000036  3FFE     SIMR

#### UNIX Register File

The following is an excerpt from a UNIX .REG file named def5206.reg (remember, UNIX is case sensitive).  UNIX detail files do not currently allow comments.

ESTSIM

SC   MBAR        00000C0F  10000001 SIMR

SC   RAMBAR   00000C04  00000000 SIMR

SC   SIMR          10000003  C0       SIMR

SC   ICR1          10000014  04       SIMR

SC   ICR2          10000015  08       SIMR

SC   ICR3          10000016  0C       SIMR

SC   ICR4          10000017  10       SIMR

SC   ICR5          10000018  14       SIMR

SC   ICR6          10000019  18       SIMR

SC   ICR7          1000001A  1C       SIMR

SC   ICR8          1000001B  1C       SIMR

SC   ICR9          1000001C  80       SIMR

SC   ICR10         1000001D  80       SIMR

SC   ICR11         1000001E  80       SIMR

SC   ICR12         1000001F  00       SIMR

SC   ICR13         10000020  00       SIMR

SC   IMR           10000036  3FFE     SIMR

### Symbol File Example

#### UNIX Symbol File

The following is an excerpt from a UNIX .SYM file named def801.sym (remember, UNIX is case sensitive).  UNIX detail files do not currently allow comments.

ESTPIN

addr,T31..T0

addrh,T31..T16

addrl,T15..T0

data,T63..T32

datah,T63..T48

datal,T47..T32

ctrl,T95..T64

size,T81..T80

at,T91..T88

av,T93..T92

a31,T0

a30,T1

### Help File Examples

#### PC Help File

PC help files are currently created using RoboHelp.  These files have a .HLP extension; for an example, use Windows Explorer to maneuver to the C:\EST32\SIMFILES subdirectory on your PC.  Locate files with an .HLP extension.  You can double-click on a file to open it in a popup window.

The resulting popup is the main entry point for the associated processor’s help file.  The hierarchical structure of the various PC SIM-related help files is explained in the section of this document entitled “Creating PC Help Files For SIM.”

#### UNIX Help File

UNIX help files are created using the UNIX Help Manager.  These files have a .help extension.  Basically, UNIX help files are simple text files with specific formatting tags applied to force the information to display properly in the visionXP or visionXD application.  The structure of the various UNIX SIM-related help files is explained in the section of this document entitled “Creating UNIX Help Files For SIM.” .   UNIX help files allow comments; preceded comment lines with two forward slashes (//).  Comment entries are not processed by the SIM application.

//

// Copyright (c) EST Corporation, 1998. All Rights Reserved.

//

// File:    Cpu.mcf5206.help

// Project: EST Utilities Motif GUI

// Desc:    Help file for MCF5206 CPU

//

// $Header: /RCS/archive/est/ti/help/Cpu.mcf5206.help,v 1.0 1998/01/29 09:29:25 stew Exp $

//

//

//

//

<volume #ESTCPU5206 MCF5206 Internal Register Guide>

//

// TOC: MCF5206 Reg.

//

__SimpleTOCEntry(1,CPU5206,MCF5206 Internal Registers)

//

This topic lists help topics available for the

MCF5206 Family Internal Registers.

To learn how to use Help, press the Help key or

choose "Using Help - Overview..." from the Help menu.

<sp>

<h2 #5206_SIMR System Integration Module (SIMR)>

<sp>

<h2 #5206_TIMER Timer Registers>

<sp>

<h2 #5206_DRAMC DRAM Controller Registers>

<sp>

<h2 #5206_UART Universal Asynchronous / Synchronous Receiver / Transmitter Module 1 (UART1) Registers>

<sp>

<h2 #5206_UART Universal Asynchronous / Synchronous Receiver / Transmitter Module 2 (UART2) Registers>

<sp>

<h2 #5206_GPIO Parallel Port (General Purpose I/O) - GPIO Registers>

<sp>

<h2 #5206_MBUS M-Bus Registers>

<sp>

<h2 #5206_CS Chip Select Registers>

<sp>

//

__SimpleTOCEntry(2,5206_SIMR,System Integration Module (SIMR))

<sp>

The System Integration Module (SIMR) register group contains the following registers:

<sp>

<bullet #5206_ACR>ACR0/ACR1 - Access Control Register

<bullet #5206_CACR>CACR - Cache Control Register

<bullet #5206_MBARR>MBAR - Module Base Address Register

<bullet #5206_RAMBAR>RAMBAR - SRAM Base Address Register

<bullet #5206_SIMRR>SIMR - SIM Configuration Register

<bullet #5206_ICR>ICR1-13 - Interrupt Control Registers

<bullet #5206_IMR>IMR - Interrupt Mask Register

<bullet #5206_IPR>IPR - Interrupt Pending Register

<bullet #5206_RSR>RSR - Reset Status Register

<bullet #5206_SYPCR>SYPCR - System Protection Control Register

<bullet #5206_SWIVR>SWIVR - Software Watchdog Interrupt Vector Register

<bullet #5206_SWSR>SWSR - Software Watchdog Service Register

<bullet #5206_PAR>PAR - Pin Assignment Register

<sp>

//

//

//

__SimpleTOCEntry(2,5206_ACR,Access Control Register (ACR0/ACR1))

//

The Access Control Registers (ACR0, ACR1) provide a definition of memory reference attributes for two memory

regions (one per ACR). This set of effective attributes is defined for every memory reference using the ACRs

or the set of default attributes contained in the CACR.  The ACRs are examined for every memory reference that

is NOT mapped to the SRAM module.

## Chip Select Information

The first part of each detail file contains the Chip Select Overview (CSVIEW) and Chip Select Detail (CSDTL) sections.  The formatting of these files for the different platforms varies slightly.  In general, if you are creating a new .DTL file for a new processor, it is best to simply copy and modify as necessary the CSVIEW and CSDTL sections of a like processor’s detail file.  For example, if you take a look at the CSVIEW and CSDTL sections of any two files for the same processor family (such as the MCF5204.DTL and MCF5206.DTL files), you will notice a great deal of similarity between the two.

### CSVIEW

The CSVIEW section of the register detail file is formatted similarly to the following example:

600,8,CS0,CS1,CS2,CS3,CS4,CS5,CS6,CS7,
900,BaseAddr,0,1,21,2,H,8,
900,AddrMask,1,5,17,2,H,7,
300,FC,0,22,4,2,H,1,
400,FCM,1,22,4,2,H,1,
600,BR6-3,0,26,4,1,B,
400,Prty,0,30,1,3,O,off,on,
400,WP,0,31,1,3,O,off,on,
500,Valid,0,32,1,3,O,no,yes,
500,Waits,1,1,4,17,C,FT,0,1,2,3,4,5,6,7,8,9,10,11,12,13,14,
500,Page,1,29,1,3,O,off,on,
600,Size,1,30,2,5,C,32 Bit,16 Bit,8 Bit,Extern,
600,Type,1,32,1,3,O,SRAM,DRAM,
BR0,OR0,BR1,OR1,BR2,OR2,BR3,OR3,BR4,OR4,BR5,OR5,BR6,OR6,BR7,OR7,

#### Chip Select Overview Section – CSVIEW

The Chip Select Overview section provides the information needed by SIM to display the target’s and the emulator’s Chip Select Table.  The Chip Select Overview Table in basic text format can be displayed by typing a CS command at the emulator’s >BKM> prompt.  The screenshot below displays the Chip Select Overview Table for a 68360.

The associated CSVIEW information in the register detail file would look like:

CSVIEW,14
600,8,CS0,CS1,CS2,CS3,CS4,CS5,CS6,CS7,
900,BaseAddr,0,1,21,2,H,8,
900,AddrMask,1,5,17,2,H,7,
300,FC,0,22,4,2,H,1,
400,FCM,1,22,4,2,H,1,
600,BR6-3, 0,26,4,1,B,
400,Prty,0,30,1,3,O,off,on,
400,WP,0,31,1,3,O,off,on,
500,Valid,0,32,1,3,O,no,yes,
500,Waits,1,1,4,17,C,FT,0,1,2,3,4,5,6,7,8,9,10,11,12,13,14,
500,Page,1,29,1,3,O,off,on,
600,Size,1,30,2,5,C,32 Bit,16 Bit,8 Bit,Extern,
600,Type,1,32,1,3,O,SRAM,DRAM,
BR0,OR0,BR1,OR1,BR2,OR2,BR3,OR3,BR4,OR4,BR5,OR5,BR6,OR6,BR7,OR7,

The CSVIEW section provides the appropriate information to properly format and display the Chip Select Table maintained by the emulator.  If you click on the Chip Select Table button in the main SIM grid, the Chip Select Overview Table will display in its GUI format, as depicted in the screenshot below:

Likewise, the same results will occur on the UNIX platform if you click on the Chip Selects... button:

Each of the lines and line components that make up the CSVIEW section are described in the following section.  The text from the sample 68360 CSVIEW section above is indicated in bold for your reference.

Line 1:

CSVIEW,14
keyword, number of option lines that follow

where:

keyword = CSVIEW

number of option lines that follow = number of option lines following first line

This line is not comma terminated.

Line 2:

600,8,CS0,CS1,CS2,CS3,CS4,CS5,CS6,CS7,
column width, number of chip selects, chip select mnemonics,

where:

column width = number indicating width of column in table

number of chip selects = number of chip select mnemonics that follow

chip select mnemonics = mnemonics for all chip selects

This line is comma terminated.

The ‘column width’ is a number determined empirically by viewing the resultant table in SIM. A good starting point is the numerical value listed in the example.  In most cases, you can copy the CSVIEW information from a like processor file and modify as necessary.

The ‘number of chip selects’ is a decimal number and indicates the number of rows in the Chip Select Table.  Each row in the Chip Select Table contains the parameters for the specific chip select. The column headings for the fields that make up the row are specified in the CSVIEW options following line 2 (the exception being the last row; which lists the Base and Option registers).

Lines 3…n

900,BaseAddr,0,1,21,2,H,8,
900,AddrMask,1,5,17,2,H,7,
300,FC,0,22,4,2,H,1,
400,FCM,1,22,4,2,H,1,
600,BR6-3, 0,26,4,1,B,
400,Prty,0,30,1,3,O,off,on,
400,WP,0,31,1,3,O,off,on,
500,Valid,0,32,1,3,O,no,yes,
500,Waits,1,1,4,17,C,FT,0,1,2,3,4,5,6,7,8,9,10,11,12,13,14,
500,Page,1,29,1,3,O,off,on,
600,Size,1,30,2,5,C,32 Bit,16 Bit,8 Bit,Extern,
600,Type,1,32,1,3,O,SRAM,DRAM,

Following Line 2, is a line for each of the chip select options. The number of lines should correspond to the number specified in line 2, second position.  The options associated with a chip select are the column headings in the Chip Select Table.  In the above example, the options are highlighted as follows:

900,Base Addr,0,1,21,2,H,8,
900,Addr Mask,1,5,17,2,H,7,
300,FC,0,22,4,2,H,1,
400,FCM,1,22,4,2,H,1,
600,BR6-3, 0,26,4,1,B,
400,Prty,0,30,1,3,O,off,on,
400,WP,0,31,1,3,O,off,on,
500,Valid,0,32,1,3,O,no,yes,
500,Waits,1,1,4,17,C,FT,0,1,2,3,4,5,6,7,8,9,10,11,12,13,14,
500,Page,1,29,1,3,O,off,on,
600,Size,1,30,2,5,C,32 Bit,16 Bit,8 Bit, Extern,
600,Type,1,32,1,3,O,SRAM,DRAM,

Each option line represents a column in the Chip Select Table.  The parameters provided in the option row specify how the column is formatted by visionSIM. Each line should be comma terminated.

Each option line can be categorized as one of the following format option types:

- Hex

- Binary

- Option

- Combo

These options and examples of them are detailed in the following section.

Last Line:

BR0,OR0,BR1,OR1,BR2,OR2,BR3,OR3,BR4,OR4,BR5,OR5,BR6,OR6,BR7,OR7,

The last line contains the Base Register / Option Register pairs that make up each chip select.  The registers are listed in the order of the chip selects, as provided in line 2.  The example above has the following line 2/last line entries:

600,8,CS0,CS1,CS2,CS3,CS4,CS5,CS6,CS7

…..

…

.

BR0,OR0,BR1,OR1,BR2,OR2,BR3,OR3,BR4,OR4,BR5,OR5,BR6,OR6,BR7,OR7

The last line is comma terminated.

#### CSVIEW Format Options

This section details the four available format options (Hex, Binary, Option, Combo) that can be specified in the CSVIEW section.

Hex Format Option:

The Hex Format Option has the following syntax:

Width, Caption,  Register, Starting Position, # of Bits, H, # of Hex Digits

where:

Width:	Indicates the relative width of the column.

Caption:  A string displayed at the top-most column position.

Register:  A ‘0’ indicates the option is specified in the Base Register; a ‘1’ indicates the option is specified in the Option Register.

Starting Position: Indicates the starting bit position for the related option bit field in the Option Register or Base Register (Most Significant Bit – MSB - is bit 1).

# of Bits:  The decimal number of bits required for the option.

H:  The ‘H’, in the final position, indicates the option line is a Hex Format Option.

# of Hex Digits:  Decimal number of Hex digits required. This is related to the # of Bits field.

This option line should be comma terminated.

Binary Format Option:

The Binary Format Option has the following syntax:

Width, Caption, Register, Starting Position, # of Bits, B,

where:

Width:	Indicates the relative column width.

Caption:  A string displayed at the top-most column position.

Register:  A ‘0’ indicates the option is specified in the Base Register; a ‘1’ indicates the option is specified in the Option Register.

Starting Position:  Indicates the starting bit position for the related option bit field in the Option Register or Base Register (Most Significant Bit – MSB - is bit 1).

# of Bits:  The decimal number of bits required for the option.

B:  The ‘B’, in the final position, indicates the option line is a Binary Format Option.

This option line should be comma terminated.

Option Format Option:

The Option Format Option has the following syntax:

Width, Caption, Register, Starting Position, # of Bits, O, 0 Caption, 1 Caption,

where:

Width:  Indicates the relative width of the column.

Caption:  A string displayed at the top-most column position.

Register:  A ‘0’ indicates the option is specified in the Base Register; a ‘1’ indicates the option is specified in the Option Register.

Starting Position:  Indicates the starting bit position for the related option bit field in the Option Register or Base Register (Most Significant Bit – MSB - is bit 1).

# of Bits:  The decimal number of bits required for the option.

O:  The letter ‘O’ indicates the Option Format Option.

0 Caption:  The string associated with the binary 0 caption.

1 Caption:  The string associated with the binary 1 caption.

This option line should be comma terminated.

Combo Format Option:

The Combo Format Option has the following syntax:

Width, Caption,  Register, Starting Position, # of Bits,C,# of elements,element 1,..,

where:

Width:  Indicates the relative width of the column.

Caption:  A string displayed at the top-most column position.

Register:  A ‘0’ indicates the option is specified in the Base Register; a ‘1’ indicates the option is specified in the Option Register.

Starting Position:  Indicates the starting bit position for the related option bit field in the Option Register or Base Register (Most Significant Bit – MSB - is bit 1).

# of Bits:  The decimal number of bits required for the option.

C:  The letter ‘C’ indicates the Combo Format Option.

# of elements:  The number of active elements.

element 1:  First element (binary 0 element).

element n:  Last element (binary 11..1 element).

This option line should be comma terminated.

UNIX Note

One formatting difference in the CSVIEW section of the detail file between the PC and UNIX versions of the file is that for the UNIX file, you must remove the extra digit that appears just before the Option Format type in each line.

900,BaseAddr,0,1,21,2,H,8,
900,AddrMask,1,5,17,2,H,7,
300,FC,0,22,4,2,H,1,
400,FCM,1,22,4,2,H,1,
600,BR6-3, 0,26,4,1,B,
400,Prty,0,30,1,3,O,off,on,
400,WP,0,31,1,3,O,off,on,
500,Valid,0,32,1,3,O,no,yes,
500,Waits,1,1,4,17,C,FT,0,1,2,3,4,5,6,7,8,9,10,11,12,13,14,
500,Page,1,29,1,3,O,off,on,
600,Size,1,30,2,5,C,32-Bit,16-Bit,8-Bit,Extern,
600,Type,1,32,1,3,O,SRAM,DRAM,

Remove the items in bold from the PC version of the CSVIEW section of the file to make it function properly on the UNIX machine.

For “C” – Combo Option types, you will also need to add the number of elements just after the “C” in the line, as follows:

900,BaseAddr,0,1,21,L,8,
900,AddrMask,1,5,17,L,8,
300,FC,0,22,4,H,1,
400,FCM,1,22,4,H,1,
600,BR6-3, 0,26,4,B,
400,Prty,0,30,1,O,off,on,
400,WP,0,31,1,O,off,on,
500,Valid,0,32,1,O,no,yes,
500,Waits,1,1,4,C,16,FT,0,1,2,3,4,5,6,7,8,9,10,11,12,13,14,
500,Page,1,29,1,O,off,on,
600,Size,1,30,2,C,4,32-Bit,16-Bit,8-Bit,Extern,
600,Type,1,32,1,O,SRAM,DRAM,

Any fields that are address-related (for example, BaseAddr and AddrMask) should be left-justified in 8 Hex nibbles to make them look like addresses.  Therefore, their type has been changed to “L,” which represents LeftHex.  This format type is only available on the UNIX platform.

If you are not familiar with the concept of nibbles, they are equivalent to half a byte, or four bits.

For “O” description lines (Option Format), you may change the format type to change the way the information displays on the screen.  If the options are quite lengthy, it’s best to use the “O” format and if they are rather short, to use the “X” format.  The alternative format, “X”, displays options side by side, as follows:

       Valid Bit             Invalid Bit

Whereas the “O” format displays options one after another, as follows:

       Negate Timing QUICC
	       Normal

This is a helpful workaround for the screen dimension limitation.

### CSDTL

The CSDTL section of the register detail file follows the CSVIEW and is formatted similarly to the following example:

CSDTL,16
O,0,Valid Bit,Valid,Invalid,32,
O,0,Write Protect,Enabled,Disabled,31,
O,0,Parity Checking Enable,Enabled,Disabled,30,
O,0,/CS Negate Timing QUICC,normal,1 phs early,29,
O,0,/CS Negate Timing EC040,normal,1 phs early,28,
O,0,Burst Acknowledge EC040,ACK EC040,NO ACK,27,
O,0,Timing Relax,Relax,No relax,26,
T,0,Function Code,Binary,22,4,
T,0,Base Address,Hex,1,21,8,
O,1,Dynamic RAM Select,DRAM,SRAM,32,
C,1,SRAM Port Size,30,2,4,32-Bit,16-Bit,8-Bit,External DSACKx,
O,1,Page Mode Enabled,Enabled,Disabled,29,
C,1,Burst Length Cycles,26,2,4,1 clock,2 clocks,3 clocks,4 clocks,
T,1,Function Code Mask,Binary,22,4,
T,1,Address Mask,Hex,5,17,7,
C,1,Cycle Length (Waits),1,4,16,Fast Termination,0 wait,1 waits,2 waits,3 waits,4 waits,5 waits,6 waits,7 waits,8 waits,9 waits,10 waits,11 waits,12 waits,13 waits,14 waits,

#### Chip Select Detail Section – CSDTL

The Chip Select Detail section describes the format and syntax of the Detail Configuration Control / Screen. The detail configuration screen is typically displayed when a chip select information field is double-clicked in the Chip Select Overview Table.

Since the chip-selects for 68K processors are made up of both a Base Register (BR) and an Option Register (OR), both the BR and OR detail windows appear.

In the above case, the detail windows are displayed if any option in the Chip Select 0 row is selected.

Likewise, the detail display for the UNIX platform is invoked if you double-click any option within the Chip Select row:

#### CSDTL Format Options

This section describes the format and syntax of the CSDTL section of the Chip Select Detail File. Each of the lines and line components that make up the CSDTL section are described in the following section.  The text from the sample 68360 CSDTL section above is indicated in bold for your reference.

Line 1:

CSDTL,16
keyword, number of option lines that follow

where:

keyword:  CSVIEW

number of option lines:  number of option lines following the first line

This line is not comma terminated

Lines 2…n:

O,0,Valid Bit,Valid,Invalid,32,
O,0,Write Protect,Enabled,Disabled,31,
O,0,Parity Checking Enable,Enabled,Disabled,30,
O,0,/CS Negate Timing QUICC,normal,1 phs early,29,
O,0,/CS Negate Timing EC040,normal,1 phs early,28,
O,0,Burst Acknowledge EC040,ACK EC040,NO ACK,27,
O,0,Timing Relax,Relax,No relax,26,
T,0,Function Code,Binary,22,4,
T,0,Base Address,Hex,1,21,8,
O,1,Dynamic RAM Select,DRAM,SRAM,32,
C,1,SRAM Port Size,30,2,4,32-Bit,16-Bit,8-Bit,External DSACKx,
O,1,Page Mode Enabled,Enabled,Disabled,29,
C,1,Burst Length Cycles,26,2,4,1 clock,2 clocks,3 clocks,4 clocks,
T,1,Function Code Mask,Binary,22,4,
T,1,Address Mask,Hex,5,17,7,
C,1,Cycle Length (Waits),1,4,16,Fast Termination,0 wait,1 waits,2 waits,3 waits,4 waits,5 waits,6 waits,7 waits,8 waits,9 waits,10 waits,11 waits,12 waits,13 waits,14 waits,

Following Line 1 is a line for each of the chip select detail options. The number of lines should correspond to the number specified in line 1, second position.

Each option line represents a line of detail.  The parameters provided in the option row specify how the detail is formatted by the SIM application. Each line should be comma terminated.

Each option line can be categorized as one of the following format option types

- Text Box

- Option Box

- Combo Box

A Text Box is used for both Hex and Binary format options.  An Option Box is used for a single bit that has two options, a binary 0 option and a binary 1 option.  Lastly, a Combo Box is used for several bits which have three or more selectable options.

Each option line is comma terminated. Remember, though, line #1 of the CSDTL section is not comma terminated.

Text Box

The Text Box format option has the following syntax:

T,Register,Caption,Format,Starting Position,# of Bits,# of Hex Digits,

where:

‘T’:  The ‘T’ specifies a Text Box option.

Register:  A ‘0’ indicates that the option is specified in the Base Register. 
		   A‘1’ indicates that the option is specified in the Option Register.

Caption:  The Caption is a string displayed at the top of the option field being 
               described.

Format:  “Hex” or “Binary”

Starting Position:  The Starting Position indicates the starting bit position for 
the related option bit field in the Option Register or Base Register. (Most Significant Bit – MSB - is bit 1).

# of Bits:  The decimal number of bits required for the option.

# of Hex Digits:  Number of Hex Digits – if Hex was specified as the Format.

The option line should be comma terminated.

Option Box

The Option Box format option has the following syntax:

O,Register,Caption,Format,Binary 1 Caption,Binary 0 Caption,Starting Position,

where:

‘O’:  The ‘O’ specifies an Option Box option.

Register:  A ‘0’ indicates that the option is specified in the Base Register. 
	                A ‘1’ indicates that the option is specified in the Option Register.

Caption:  The Caption is a string displayed at the top of the option field being 
               described.

Binary 1 Caption:  Text string for the binary 1 option.

Binary 0 Caption:  Text string for the binary 0 option.

Starting Position:   The Starting Position indicates the starting bit position for 
the related option bit field in the Option Register or Base Register. (Most Significant Bit – MSB -  is bit 1).

The option line should be comma terminated.

Combo Box

The Combo Box format option has the following syntax:

C,Register,Caption,Starting Position,# of Bits,# of Elements,element 1,…,element n,

where:

‘C’:  The ‘C’ specifies a Combo Box option.

Register:  A ‘0’ indicates that the option is specified in the Base Register. 
	  	  A ‘1’ indicates that the option is specified in the Option Register.

Caption:  The Caption is a string displayed at the top of the option field being 
                described.

Starting Position:  The Starting Position indicates the starting bit position for 
the related option bit field in the Option Register or Base Register (Most Significant Bit - MSB - is bit 1).

# of Bits:  Number of bits that make up the option.

# of Elements:  Number of elements for the specified number of bits.

Element 1:  The caption of element 1 (binary 0…0 option).

Element n:  The caption of element n (binary n option)

The option line should be comma terminated.

## Non-Chip Select Register Descriptions

#### PC Format

This section describes the syntax and format of non-chip select register descriptions.  The following example is from the MCF5204.DTL file and shows some of the registers in the SIM register group.

#

# SIM Register Group

#

(CSAR0),1,Chip-Select Address Register 0,SIM,SIM_CSARX_HELP

T,BA[9:31]-Base Address,1,23,Hex,SIM_CSARX_BA_HELP,

(CSMR0),7,Chip-Select Mask Register 0,SIM,SIM_CSMRX_HELP

T,BAM[9:31]-Base Address Mask,1,23,Hex,SIM_CSMRX_BAM_HELP,

O,WP-Write Protect,Only read access allowed,Read and Write allowed,24,SIM_CSMRX_WP_HELP,

O,C/I-CPU Space and IR Acknowledge Mask,Mask this type of transfer,Do not mask this type of transfer,27,SIM_CSMRX_C/I_HELP,

O,SC-Supervisor Code Mask,Mask this type of transfer,Do not mask this type of transfer,28,SIM_CSMRX_SC_HELP,

O,SD-Supervisor Data Mask,Mask this type of transfer,Do not mask this type of transfer,29,SIM_CSMRX_SD_HELP,

O,UC-User Code Mask,Mask this type of transfer,Do not mask this type of transfer,30,SIM_CSMRX_UC_HELP,

O,UD-User Data Mask,Mask this type of transfer,Do not mask this type of transfer,31,SIM_CSMRX_UD_HELP,

You will notice a label at the end of each detail line for non-chip select registers (for example, SIM_CSMRX_SC_HELP,).  This label provides information to the SIM application and to the RoboHelp application as to how to invoke bit-level help for a register.  Help labels are described in greater detail later in this document.

A Comment Line is prefixed with a ‘#’ character.  Remember, comment lines are not processed by the SIM application.

Note:     As a general rule of thumb, all register description sections should contain a comment line which states where the register is described in the Motorola manual, as well as the version of the manual used.  Most of the Motorola processor user manuals are posted in .PDF format on the Motorola Web site (www.mot.com).  This will make it easier to track down problems and provide the person doing the help files with the information needed to locate the appropriate information.

Each register description section consists of a first line followed by a number of option description lines.

Line #1 in the register description section specifies the top-level register information and the number of option lines that follow.  The option lines describe the individual bit fields within the register.

Line #1:

Line number one for each register is formatted as follows; the line in bold is presented as an example from the file excerpt displayed above.

(CSMR0),7,Chip-Select Mask Register 0,SIM,SIM_CSMRX_HELP
(Register Mnemonic),# of Option Lines,Register Title,Group,Help Index

where:

Register Mnemonic:  The Register Mnemonic is the official name assignment 
for the register under description. The register name should match the register name given by the firmware exactly.  The register mnemonic should be surrounded by parentheses.

# of Option Lines:  The ‘# of Option Lines’ entries indicates the number of 
lines that follow the first line.  The number is a decimal number and is used by SIM to help parse the register description field.

Register Title:  The Register Title is a string that describes the register. The 
title traditionally is the title found in the associated Motorola User Manual.

Group Mnemonic:  The Group Mnemonic specifies the group that the register 
belongs to. The Group Mnemonic should match the official groups specified in the .REG register file.

Help Index:  The Help Index is an equated label from a HELPINDEX entry line at the top of the .DTL file. The HELPINDEX on line # 1 specifies the Register Level Help.  Register Level Help is generic help and usually provides a description of the register.

Line #1 is not comma terminated.

Option Lines:

After line #1, the individual options, or register bit field descriptions, are described on separate lines. The total number of option lines was specified on line #1. An option line can be one of the following format types:

- Text Box

- Combo Box

- Option Box

Text Box Option Lines

A Text Box is used to describe a single selection field. The field can consist of any number of bits and can be described in either Binary or Hex.

The general syntax for the Text Box Option Line is as follows; the line in bold is presented as an example:

T,BAM[9:31]-Base Address Mask,1,23,Hex,SIM_CSMRX_BAM_HELP,
	T,title,starting bit position,number of bits,Hex/Binary,Hex digits,help index,

where:

T:  Indicates the option line is a Text Box Option Line. The ‘T’ should be 
capitalized.

title:  The title is a text description of the register bit/field being described. 
A good convention to follow here is to provide both the bit field mnemonic and the title, as follows:

ENb-Enable Channel B

TXe-Transmit Enable

starting bit position:  The starting bit position is the relative bit position within 
the register of the bit field being described. The relative positions are numbered from 1 to n, where 1 is always the Most Significant Bit (MSB) in the register.

Note:    Unfortunately, the starting bit position has little to do with the actual bit number in the register.  In the case of PowerPC, bit 1 is actually bit 0 in the register; in the case of 68K and ColdFire, bit 1 is actually bit 31 in the register (assuming a 32-bit register).

Therefore, for the PowerPC, bits are numbered from 0 to 31 (MSB to LSB) for a 32-bit register.  For the 68K and ColdFire, bits are numbered from 31 to 0 (MSB to LSB) for a 32-bit register.

number of bits:  The number of bits field is a decimal number that indicates the 
length of the bit field being described.

Hex/Binary:  The Hex/Binary field indicates if the entry should be display in 
Binary or Hex by SIM.

Hex Digits:  The number of Hex Digits field determines how SIM should 
display the bit field.  This field only applies if HEX was specified in the previous field. As an example, if a field is 10 bits, and is to be displayed in HEX, then the Hex Digits field must be at least 3 (it could be 4, 5, etc.)

Help Index:  The Help Index is an entry that indicates the bit-level 
help for the bit field being described. In a sense, this entry is optional; however, if no entry is specified, when users attempt to obtain help on a particular bit, they are defaulted to the register-level help for the register associated with the bit.

The Text Box line should be comma terminated.

Option Box Option Lines

An Option Box is used to describe an option that has two selectable choices – a binary 1 selection and a binary 0 selection.

The general syntax for the Option Box Option Line is:

O,WP-Write Protect,Only read access allowed,Read and Write allowed,24,SIM_CSMRX_WP_HELP,
O,title,binary 1 caption,binary 0 caption, starting bit position,help index,

where:

O:  Indicates the option line is an Option Box Line.  The ‘O’ should be 
capitalized. The letter is the alphabetic character O not zero.

title:  The title is a text description of the register bit/field being described. 
A good convention to follow here is to provide both the bit field mnemonic and title, as follows:

ENb-Enable Channel B

TXe-Transmit Enable

Binary 1 caption:  The binary 1 caption text field contains the text that is to be 
displayed by SIM when the option is set to a binary 1.

Binary 0 caption:  The binary 0 caption text field contains the text that is to be 
displayed by visionSIM when the option is set to a binary 0.

starting bit position:  The starting bit position is the relative bit position within 
the register, of the bit field being described. The relative positions are numbered from 1 to n, where 1 is always the Most Significant Bit (MSB) in the register.

Note:    Unfortunately, the starting bit position has little to do with the actual bit number in the register. In the case of PowerPC, bit 1 is actually bit 0 in the register; in the case of 68K, bit 1 is actually bit 31 in the register (assuming a 32 bit register).

Help Index: The Help Index is an entry that indicates the bit-level 
help for the bit field being described.  In a sense, this entry is optional; however, if no entry is specified, when users attempt to obtain help on a particular bit, they are defaulted to the register-level help for the register associated with the bit.

The Option Box line should be comma terminated.

Combo Box Option Lines

A Combo Box Option is used to describe a field with three or more options. An example of a possible use of this type of option line is a field consisting of three bits with eight distinct selections.

The general syntax for the Combo Box Option Line is:

C,PS-IRQ 4 & IRQ 1 Controlled Port Size,25,2,2,32-bit D[0:31],8-bit 
D[24:31],16-bit D[16:31],16-bit D[16:31],SIM_CSCRX_PS_HELP,
C,title,starting bit position,number of bits,# elements,element1, ... ,help index,

where:

C:  Indicates the option line is a Combo Box Option Line. The ‘C’ should be 
capitalized.

title:  The title is a text description of the register bit/field being described. A 
good convention to follow here is to provide both the bit field mnemonic and title, as follows:

ENb-Enable Channel B

TXe-Transmit Enable

starting bit position:  The starting bit position is the relative bit position within 
the register, of the bit field being described. The relative positions are numbered from 1 to n, where 1 is always the Most Significant Bit (MSB) in the register.

Note:    Unfortunately, the starting bit position has little to do with the actual bit number in the register. In the case of PowerPC, bit 1 is actually bit 0 in the register; in the case of 68K, bit 1 is actually bit 31 in the register (assuming a 32-bit register).

number of bits:  The number of bits field is a decimal number that indicates the 
length of the bit field being described.

# of elements:  This field indicates the number of elements described. The 
number of elements relates to the number of bits. For example, if the number of bits is 3, there are 8 possible elements. If the option being described has only 6 elements, it still requires 3 bits. In this case, the number of bits would be specified as 3 and the number of elements would be specified as 6.

element 1:  The element 1 is the text capture for the binary 0 element of the 
element list.

element n:  The element n is the text capture for the binary n-1 element of the 
element list.

Note:    Elements must be listed in ascending binary order. If there are any holes, the bits should be listed as reserved. As an example, if  00=something 10=something, and 11=something, then 01 should be listed as 01=reserved.

Help Index: The Help Index is an entry that indicates the bit-level 
help for the bit field being described.  In a sense, this entry is optional; however, if no entry is specified, when users attempt to obtain help on a particular bit, they are defaulted to the register-level help for the register associated with the bit.

The Combo Box line should be comma terminated.

Formatting UNIX Detail Files

Detail files for the UNIX platform must begin with the word ESTDTAIL, followed by two spaces, in order to function properly.

ESTDTAIL

CSVIEW,14

600,8,CS0,CS1,CS2,CS3,CS4,CS5,CS6,CS7,

900,BaseAddr,0,1,21,L,8,

900,AddrMask,1,5,17,L,8,

300,FC,0,22,4,H,1,

400,FCM,1,22,4,H,1,

600,BR6-3,0,26,4,B,

400,Prty,0,30,1,O,off,on,

400,WP,0,31,1,O,off,on,

500,Valid,0,32,1,O,no,yes,

500,Waits,2,1,4,C,16,FT,0,1,2,3,4,5,6,7,8,9,10,11,12,13,14,

500,Page,2,29,1,O,off,on,

600,Size,2,30,2,C,4,32-Bit,16-Bit,8-Bit,Extern,

600,Type,2,32,1,O,SRAM,DRAM,

In addition, for the PowerPC and ColdFire processors only, you will need to modify detail lines with types of “O” or “X” as follows.  In the UNIX version of these detail files, the starting bit parameter should be in the third position of the detail line, followed by a “1” in the fourth position.  In the PC version of these detail files, the starting bit parameter is in the fifth position in the detail line.

## Register Files

The .REG files are simple text files.  The information that is contained in the .REG file is the same information displayed when you type an SC (System Configuration) emulator command at a >BKM> prompt.

It is important that the register listing in the .REG file match the firmware, so take special care to match register names in the two locations.  It is also important that the register listing in the .REG file match the registers listed in the Title position of the detail lines in the .DTL file.  There is a UNIX script called checkreg.pl that can be run to compare the contents of the .DTL file with the registers listed in the .REG file.

Both the PC and UNIX version of the .REG file is essentially the same, with one small difference.

The beginning of the .REG file for UNIX should start with the word ESTSIM and then two spaces; without this text and these spaces, the file will not function properly.

For example,

ESTSIM

SC   MBAR             00000C0F  10000001 SIMR

SC   RAMBAR        00000C04  00000000 SIMR

SC   SIMR               10000003  C0            SIMR

SC   ICR1                10000014  04            SIMR

SC   ICR2                10000015  08            SIMR

## Symbol Files

Symbol files are only used by the UNIX version of SIM.  You can obtain a symbol file from Engineering; this file is a listing of pin symbols for the associated processor.

The Symbol File must begin with the word ESTPIN, followed by two spaces; without this text and these spaces, the file will not function properly.

ESTPIN

addr,T31..T0

addrh,T31..T16

addrl,T15..T0

data,T63..T32

datah,T63..T48

datal,T47..T32

ctrl,T95..T64

size,T81..T80

at,T91..T88

av,T93..T92

a31,T0

## Creating PC Help Files For SIM

This section provides information on the associated PC help files for the SIM product.  On the PC, help files are named exactly as the detail files are, except with an .HLP extension (for example, 68360.HLP).

A help file provides the register and bit-level help for each register within a processor family.  In an ongoing effort to provide the most comprehensive bit-level help for our customers, and to reduce the amount of time they spend thumbing through bulky reference manuals for bit information, the help files are typically created in parallel to the .DTL files.  As each HELPINDEX label is used in the detail file, the corresponding help topic is created in the help file.  Likewise, when detail files are updated with information from new or revised processor manuals, the corresponding help file should also be updated.

For new processors, the easiest method is to create the PC version of the help files first, then cut and paste the material into the UNIX version of the help file.  Since the formatting of the two files is different, there is some additional labor involved in porting a PC help file to UNIX.

Currently, an automated GUI help file generator called Help Yourself! Is used to create the PC versions of the help files.  These files will be converted to RoboHelp sometime in the future.  To save yourself from having to learn two separate help systems, you may just want to begin porting existing help files to RoboHelp and create all new help files in RoboHelp.

Before describing the contents of a help file, an actual example might help clarify the concepts. Below is an excerpt from the MPC8I2C.DTL and MPC8I2C.HLP files.  The excerpts are for the register level and bit-level help on the I2MOD register.

Detail Description of I2MOD Register

In the following example, each detail line ends with a help index that corresponds to a topic within the .HLP file.

(I2MOD),5,I2C Mode Register,I2C,I2C_I2MOD_HELP
O,REVD-Reverse Data,3,1,Normal,Reverse Data,I2C_I2MOD_REVD_HELP,
O,GCD-General Call Disable,4,1,General Call Address is Disabled,General 
Call Address is  Enabled,I2C_I2MOD_GCD_HELP,
O,FLT-Clock Filter,4,1,I2CLK is filtered by a digital filter,I2CLK is not filtered,I2C_I2MOD_FLT_HELP,
C,PDIV-Pre Divider,6,2,4,Use the BRGCLK/32 as input to I2C BRG,Use the BRGCLK/16 as input to I2C BRG,Use the BRGCLK/8 as input to I2C BRG,Use the BRGCLK/4 as input to I2C BRG,I2C_I2MOD_PDIV_HELP,
O,EN-Enable I2C,8,1,I2C is Enabled,I2C is Disabled,I2C_I2MOD_EN_HELP,

Help File Register and Bit-Level Help for I2MOD Register / REVD Bit

I 2 C Mode Register

The I 2 C mode (I2MOD) register is a read/write register cleared at reset that controls both the I 2 C operation mode and clock source.

REVD - Reverse Data Bit

The REVD bit determines the receive and transmit character bit order.

Help information is obtained from Motorola User Manuals, which are available in hardcopy form or on the Motorola Web Site (www.mot.com).  It is not necessary to include every bit of detail from the user manuals; however, we do want to provide enough information so that the user is not forced to consult the manuals, except in rare circumstances where, for example, a complex table cannot be translated into the help system.

The actual help information is created in the RoboHelp application.  However, without the help index links, the help information file is useless.  Once you become familiar with RoboHelp, you can simply look at an existing help file and use it as a template.

#### Help Indices

The start of every detail file contains a listing of help indices, as follows:

HELPINDEX,CSVIEW_HELP,100

HELPINDEX,CSDTL_HELP,200

HELPINDEX,SIM_CSARX_HELP,300

HELPINDEX,SIM_CSARX_BA_HELP,302

HELPINDEX,SIM_CSMRX_HELP,400

HELPINDEX,SIM_CSMRX_BAM_HELP,402

HELPINDEX,SIM_CSMRX_WP_HELP,404

HELPINDEX,SIM_CSMRX_C/I_HELP,406

HELPINDEX,SIM_CSMRX_SC_HELP,408

HELPINDEX,SIM_CSMRX_SD_HELP,410

HELPINDEX,SIM_CSMRX_UC_HELP,412

HELPINDEX,SIM_CSMRX_UD_HELP,414

HELPINDEX,SIM_CSCRX_HELP,500

HELPINDEX,SIM_CSCRX_WS_HELP,502

HELPINDEX,SIM_CSCRX_BRST_HELP,504

HELPINDEX,SIM_CSCRX_AA_HELP,506

The lines are formatted as follows:

HELPINDEX,Register Group Name_Register_HELP,Context ID Number

for register-level help lines, and

HELPINDEX,Register Group Name_Register_Bit_HELP,Context ID Number

for bit-level help lines.

These lines are not comma terminated.

The numbers correspond to the context identification numbers used by RoboHelp.  These number must match exactly, in order for bit-level help to function properly.  When numbering lines in the help indices, begin each new register at a multiple of 100.  Use increments of two when numbering the bit lines.  This numbering scheme will allow the inclusion of additional lines in the future without the need to renumber the entire file, if necessary.

Within the detail file, each line ends with the same help index.

For example:

HELPINDEX,SIM_CSMRX_HELP,400

Corresponds to:

(CSMR0),7,Chip-Select Mask Register 0,SIM,SIM_CSMRX_HELP

And:

T,BAM[9:31]-Base Address Mask,1,23,Hex,SIM_CSMRX_BAM_HELP,

Corresponds to:

HELPINDEX,SIM_CSMRX_BAM_HELP,402

Remember that only the bit-level description lines are comma terminated.

It is helpful to print out the help indices to compare to your detail description lines and help application file; you’ll find that it is frustrating to find that help is not invoked in SIM simply because you mis-numbered or mis-typed a help index.

## Creating UNIX Help Files For SIM

This section provides information on the associated UNIX help files for the SIM product.  On the UNIX, help files are named as follows:

Cpu.processorname.help

For example, Cpu.mpc801.help, Cpu.68360.help, Cpu.mcf5204.help.

Remember, UNIX is case sensitive, so it is important to use the convention listed above when naming your files.

Just as with the PC help files, the UNIX help files are typically created in parallel to the .DTL files. Likewise, when detail files are updated with information from new or revised processor manuals, the corresponding help file should also be updated.  Unlike the PC help files, however, there are no help indices within the detail file.

For new processors, the easiest method is to create the PC version of the help files first, then cut and paste the material into the UNIX version of the help file.  Since the formatting of the two files is different, there is some additional labor involved in porting a PC help file to UNIX.  It is also important to remember that there are no help indices in the UNIX version of the help files, so you will have to manually strip them out.

UNIX help files are created in one of two ways: either in a PC text editor, or on a UNIX machine using vi or another editor.  If you are familiar with UNIX, vi is sufficient.  However, if you are more comfortable in a PC editing environment, it is simple enough to create your text file there, add the necessary Help Manager tags, and then port the file to the UNIX machine.

As you can see, there is a little more to the creation of UNIX help files than to PC help files; before describing the contents of a UNIX help file, an actual example might help clarify the concepts. Below is an excerpt from the reg5206.dtl and Cpu.mcf5206.help files.  The excerpts are for the register level and bit-level help on the ACR (Access Control) register.

Detail Description of Access Control Register

(ACR0),7,Access Control Register 0

T,AB-Address Base,1,8,LeftHex,2,

T,AM-Address Mask,9,8,LeftHex,2,

X,EN-Enable,17,1,ACR0 enabled,ACR0 disabled,

C,SM-Supervisor Mode,18,2,4,Match if user mode,Match if supervisor,Match always,Match always,

O,CM-Cache Mode,26,1,Disabled,Enabled,

X,BWE-Buffered Write Enable,27,1,Write,Don't Write,

X,WP-Write Protect,30,1,Only Read Permitted,Read/Write Permitted,

Help File Register and Bit-Level Help for I2MOD Register / REVD Bit

//

// Copyright (c) EST Corporation, 1998. All Rights Reserved.

//

// File:    Cpu.mcf5206.help

// Project: EST Utilities Motif GUI

// Desc:    Help file for MCF5206 CPU

//

// $Header: /RCS/archive/est/ti/help/Cpu.mcf5206.help,v 1.0 1998/01/29 09:29:25 stew Exp $

//

//

//

//

<volume #ESTCPU5206 MCF5206 Internal Register Guide>

//

// TOC: MCF5206 Reg.

//

__SimpleTOCEntry(1,CPU5206,MCF5206 Internal Registers)

//

This topic lists help topics available for the

MCF5206 Family Internal Registers.

To learn how to use Help, press the Help key or

choose "Using Help - Overview..." from the Help menu.

<sp>

<h2 #5206_SIMR System Integration Module (SIMR)>

<sp>

<h2 #5206_TIMER Timer Registers>

<sp>

<h2 #5206_DRAMC DRAM Controller Registers>

<sp>

<h2 #5206_UART Universal Asynchronous / Synchronous Receiver / Transmitter Module 1 (UART1) Registers>

<sp>

<h2 #5206_UART Universal Asynchronous / Synchronous Receiver / Transmitter Module 2 (UART2) Registers>

<sp>

<h2 #5206_GPIO Parallel Port (General Purpose I/O) - GPIO Registers>

<sp>

<h2 #5206_MBUS M-Bus Registers>

<sp>

<h2 #5206_CS Chip Select Registers>

<sp>

//

__SimpleTOCEntry(2,5206_SIMR,System Integration Module (SIMR))

<sp>

The System Integration Module (SIMR) register group contains the following registers:

<sp>

<bullet #5206_ACR>ACR0/ACR1 - Access Control Register

<bullet #5206_CACR>CACR - Cache Control Register

<bullet #5206_MBARR>MBAR - Module Base Address Register

<bullet #5206_RAMBAR>RAMBAR - SRAM Base Address Register

<bullet #5206_SIMRR>SIMR - SIM Configuration Register

<bullet #5206_ICR>ICR1-13 - Interrupt Control Registers

<bullet #5206_IMR>IMR - Interrupt Mask Register

<bullet #5206_IPR>IPR - Interrupt Pending Register

<bullet #5206_RSR>RSR - Reset Status Register

<bullet #5206_SYPCR>SYPCR - System Protection Control Register

<bullet #5206_SWIVR>SWIVR - Software Watchdog Interrupt Vector Register

<bullet #5206_SWSR>SWSR - Software Watchdog Service Register

<bullet #5206_PAR>PAR - Pin Assignment Register

<sp>

//

//

//

__SimpleTOCEntry(2,5206_ACR,Access Control Register (ACR0/ACR1))

//

The Access Control Registers (ACR0, ACR1) provide a definition of memory reference attributes for two memory

regions (one per ACR). This set of effective attributes is defined for every memory reference using the ACRs

or the set of default attributes contained in the CACR.  The ACRs are examined for every memory reference that

is NOT mapped to the SRAM module.

<sp>

The ACRs are 32-bit write-only supervisor control registers. They are accessed in the CPU address space via the

MOVEC instruction with an Rc encoding of $004 and $005. The ACRs can be read when in background debug mode (BDM).

At system reset, the entire registers are cleared.

<sp>

Notice the formatting of various lines.  Basically, certain pre-defined tags enclosed in either parentheses or pointy parentheses are used to define the way a certain portion of text should appear.

## Comment Lines (‘#’ or ‘//’)

Comments can be added to the register detail files and also to the UNIX help files.   For all files, be sure to comment and include the revision date, revision number, and changes made.  In addition, use comments to enter any information you deem necessary for others who are viewing, editing, or QA’ing the file.  Comments can be used when more than one person is working on a file, or when you are not sure about certain information and want to flag it for future reference, or for any other information you feel is necessary.

Remember that the files are simple text files and may be viewed by our customers, so keep your comments professional and to the point.

For a PC detail file, any line that begins with a ‘#’ character is considered a comment line.   There is no limit to the number of comments or comment lines in a file.  SIM does not process commented lines.

For a UNIX detail or help file, any line that begins with two forward slashes (‘//’) is considered a comment line.  There is no limit to the number of comments or comment lines in a file.  SIM does not process commented lines.

## Porting Files to UNIX

If you opt to edit detail, register, and help files for UNIX on a PC text editor, you will need to port the completed files to the UNIX machine.  This is a fairly straightforward task; it will depend, however, on how the machines are set up (network, standalone, etc.).  Determine how you can move the files between the machines.  Once a PC file has been copied to a UNIX machine, it is necessary to run a utility called DOS2UNIX on the file to strip out the <Control>/<M> end of line characters that are automatically inserted by the PC application.

To edit and work with files for the UNIX platform, it is best to get some instruction from one of the engineers who can advise you as to file placement, location, manipulation, etc.

## Job Aids

The pin-up reminder sheet on the next page is a helpful aid when coding the PC detail files; as you become more familiar with the formatting for the various processors and platforms, you may want to create similar charts for the different combinations.

- Detail Line Format for PC SIM

- Line #1:

(register abbrev.), number of option lines, register name, group name, help index

(SYPCR), 7, System Protection Register, SIU,SIU_SYPCR_HELP

- Text Box:

- T, title, starting bit position, number of bits, Hex/Binary, Hex digits, help index,

- T, DBPC Debug Pins Configuration, 12, 2, Hex, 1,SIU_DBPC_PINS_HELP,

- Option Box:

- O, title, starting bit position, 1, binary 1 caption, binary 0 caption, help index,

- O, Int 6 ED6, 1, Falling Edge, Level,COMM_MBAR_INT_HELP,

- Combo Box:

- C, title, starting bit position, number of bits, # elements, element1, ... , help index,

- C, Clock Output 1 Mode, 7, 2, 2,  element 1, element 2, SI_SIMODE_INP_HELP,